# Monday 23rd May 2022

07:45	Pressure in the chamber: 8.4x10-9 mbar

# Wednesday 25th May 2022 – Deposition at 20 K

08:00	Pressure in the chamber: 6.8x10-9 mbar

CRYO ON

Rachel James beam time for the next two weeks.

## Bkg scans

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69526 | 110-220 | 1 | 15/2 | 19.4K | 5NPLC | -300 | ON | Clean MgF2 |
| 69527 | 110-165 | 0.5 |  |  |  | -450 |  | Clean MgF2 |
| 69528 | 160-220 |  |  |  |  | -100 |  | Clean MgF2 |
| 69529 | 210-330 | 1 |  |  |  | 400 | OFF | Clean MgF2 |

Test of TimeScan, settings #av of 5 i.e. 750 ms according to the time per point of 150 ms quoted in the time scan window. However, have now noticed that this value of time per point is a fixed value assuming the PLC is in use and not the block time from the cRio which is actually being used for the measurement.

So leave the block time for the cRio to be 100 ms (as changing this would mess things up for the scanning) and then will need to tell the OU lot that the time is not correct!

## H2O dosing for 30s at 1e-7mbar

File: T69532

Temperature of substrate during deposition: 19.4K

Time start = 47s Time stop = 1m18 total dose = 31,86 dose 1-1,2x10-7 mbar for 30s

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69533 | 110-220 | 1 | 15/2 | 19.4K | 5NPLC | -300 | ON | H2O |
| 69534 | 110-165 | 0.5 | 15/2 |  |  | -450 |  | H2O |
| 69535 | 160-220 |  |  |  |  | -100 |  | H2O |
| 69536 | 210-330 | 1 |  |  |  | +400 | OFF | H2O |

## Scan after lunch – everything the same

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69537* | 110-220 | 1 | 15/2 | 19.4K | 5NPLC | -300 | ON | H2O |
| 69538 |  |  |  |  |  |  |  |  |

* Opened air valve by mistake

69539, 69540 - aborted

## Anneal H2O: 20K --- 42K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69541* | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -100 | ON | H2O |
| 69544 | 110-165 | 0.5 | 15/2 |  |  | -450 |  | H2O |
| 69545 | 160-220 |  |  |  |  | -100 |  | H2O |
| 69546 | 110-220 | 1 |  |  |  | -300 |  | H2O |

*wrong table position

69542, 69543 – aborted

## Anneal H2O: 20K --- 50K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69547 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69548 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 60K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69549 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69550 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 70K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69551 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69552 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 80K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69553 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69554 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 90K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69555 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69556 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |
| 69557 | 210-340 | 1 |  |  |  | +400 | OFF | H2O |

## Anneal H2O: 20K --- 100K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69559 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69560 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

69558– aborted

## Anneal H2O: 20K --- 110K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69561 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69562 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 120K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69563 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69564 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 130K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69565 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69566 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |

## Anneal H2O: 20K --- 140K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69567 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69568 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |
| 69569 | 110-220 | 1 |  |  |  | 400 |  | H2O |
| 69570 |  |  |  |  |  |  |  | H2O |
| 69571 |  |  |  |  |  |  |  |  |

Is 140K the onset of desorption?

## Anneal H2O: 20K --- 150K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69572 | 110-165 | 0.5 | 15/2 | 19.4K | 5NPLC | -450 | ON | H2O |
| 69573 | 160-220 | 0.5 |  |  |  | -100 |  | H2O |
| 69574 | 110-220 | 1 |  |  |  | -300 |  |  |

## Anneal H2O: 20K --- 155K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69575 | 110-220 | 0.5 | 15/2 | 19.4K | 5NPLC | -300 | ON | H2O |

## Anneal H2O: 20K --- 160K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69576 | 110-220 | 0.5 | 15/2 | 19.4K | 5NPLC | -300 | ON | H2O |

## Anneal H2O: 20K --- 170K

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69578 | 110-220 | 0.5 | 15/2 | 19.4K | 5NPLC | -300 | ON | H2O |

Onset of desorption at 140 K, fully desorbed by 170 K?

Is this band sensitive to phase change? Possible at 120K?

# Thursday 26th May 2022 – Deposition at 50 K

07:05	p = 8.8e-8 mbar	T = 273 K	cryo ON

Plan

- Deposit at 50 K

- Analyse dep 20 K results

09:22	p = 2.4e-9 mbar	T = 19. 3 K

09:40	p = 4e-9 mbar	T = 50 K

## Bkg scans at 50 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69578* | 110-220 | 0.5 | 15/2 | 19.4K | 5NPLC | -300 | ON | Clean MgF2 |
| 69579 |  |  |  | 50 K |  |  |  |  |
| 69580 | 210-220 | 1 |  |  |  | +400 | OFF | Clean MgF2 |

*wrong temperature

## H2O dosing for 30s at 1e-7mbar

File: T69583
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 50K

Time start =36 s

Time stop = 1m06

Total dose = 30s  dose 1-1,2x10-7 mbar for 30s

T69581, T69592 * not ready

| File |  range |  step | #av/#sc | Temp /K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69584 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O |

## Anneal to 60 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69585 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 70 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69586 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 80 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69587 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69588 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69589 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 110 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69590 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69591 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 130 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69592 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69593 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 150 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69594 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

## Anneal to 160 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69595 | 110-220 | 0.5 | 15/2 | 50K | 5NPLC | -300 | ON | H2O at 60 K |

# Thursday 26th May 2022 – Deposition at 100 K 1

16:25	p=9e-9	T=100 K

## Bkg scans at 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69597 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | Clean MgF2 |
| 69599 | 138-150 | 0.2 |  |  |  | -450 | OFF | Clean MgF2 |

69596, 69598 – aborted

## H2O dosing for 30s at 1e-7mbar

File: T69601
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 100K

Time start =1m06 s

Time stop = 1m36

Total dose = 30s  dose 1-1,2x10-7 mbar for 30s

T69600 aborted

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69604 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | H2O |

69603, 69603.d02 aborted

Absorbance low – dose again.

## H2O dosing for 30s at 1e-7mbar, total = 60s

File: T69605
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 100K

Time start =30 s

Time stop = 60s

Total dose = 30s  dose 1-1,2x10-7 mbar for 30s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69605 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | H2O |

No d02 - Not thick enough

## H2O dosing for 60s at 1e-7mbar, total = 120s

File: T69607
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 50K

Time start =30 s

Time stop = 60s

Total dose = 30s  dose 1-1,2x10-7 mbar for 30s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69606 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | H2O |

Doh, wrong background files. Ice too thic – although features around 195 nm now enhanced… what are these? RESTART

# Thursday 26th May 2022 – Deposition at 100 K 2

19:30	P = 1.4e-8 mbar	T = 100 K

## Bkg scans at 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69609 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | Clean MgF2 |
| 69610 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69611 | 175-205 | 0.2 |  |  |  | -100 |  |  |

## H2O dosing for 60s at 1e-7mbar

File: T69612
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 100K

Time start =36 s

Time stop = 1m36s

Total dose = 60s  dose 1-1,2x10-7 mbar for 30s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69613 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON | H2O |
| 69614 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dosing for 240s at 1e-7mbar

File: T69615
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 100K

Time start = ?

Time stop = ?

Total dose = 4m

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69616 | 175-205 | 0.5 | 15/2 | 100K | 5NPLC | -100 | ON | H2O |

## H2O dosing for 120s at 1-1.2e-6mbar

File: T69617
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 100K

Time start =23 s

Time stop = 2m21s

Total dose = 120s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69618 | 175-205 | 0.5 | 15/2 | 100K | 5NPLC | -100 | ON | H2O |

## Anneal to 136 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69619 | 175-205 | 0.5 | 15/2 | 136K | 5NPLC | -100 | ON | H2O |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69620 | 175-205 | 0.5 | 15/2 | 140K | 5NPLC | -100 | ON | H2O |

## Anneal to 150-160 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69621 | 175-205 | 0.5 | 15/2 | 150-160K | 5NPLC | -100 | ON | H2O |

## Anneal to 160 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69622 | 175-205 | 0.5 | 15/2 | 160K | 5NPLC | -100 | ON | H2O |

Desorbing.

# Friday 27th May 2022 – Deposition at 60 K

Morning Nyk,

Hope the gardening went well. Could you stick the cryo on?

Thanks, Rachel

07:45	p = 1.8e-8 mbar	T = 271. K

cryo ON

09:10	P = 6.e-9 mbar	T = 60 K

## Bkg scans at 60 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69623 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON | Clean MgF2 |
| 69624 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## H2O dosing for 30s at 1e-7mbar

File: T69625
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 60K

Time start = 23 s

Time stop =

Total dose = 30s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69626 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON |  |
| 69627 | 138-150 | 0.2 |  |  |  | -450 |  |  |
| 69628 | 138-150 | 0.2 |  |  |  | -450 |  |  |

*Wrong scan range

Are there any peak position shifts due to thickness?

## H2O dosing for 15s at 1e-7mbar

File: T69629
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 60K

Time start = 1m16s

Time stop = 1m32s

Total dose = 45s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69630* 69631 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON |  |
| 69632 69633 | 138-150 | 0.2 |  |  |  | -450 |  |  |

*aborted

## H2O dosing for 15s at 1e-7mbar

File: T69634
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 60K

Time start = 16s

Time stop = 26s

Total dose = 60s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69635* 69636 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON |  |

*aborted

## Anneal to 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69637 | 110-220 | 0.5 | 15/2 | 100K | 5NPLC | -300 | ON |  |

## Anneal to 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69638 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON |  |

## Anneal to 130 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69639 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON |  |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69640 | 110-220 | 0.5 | 15/2 | 140K | 5NPLC | -300 | ON | H2O |

PC d32519

## Anneal to 230 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69641* 69642 | 110-220 | 0.5 | 15/2 | 230K | 5NPLC | -300 | ON | H2O |

*aborted

# Friday 27th May 2022 – Deposition at 40 K

## BKG scans at 40 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69643 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON | Clean MgF2 |
| 69644 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69645 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69646
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 40K

Time start =  37

Time stop = 52

Total dose = 15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69647 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON | H2O |
| 69648 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69649
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 40K

Time start =  37

Time stop = 52

Total dose = 15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69650 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON | H2O |
| 69651* 69652 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 14s at 1e-7mbar

File: T69653
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 40K

Time start =  29

Time stop = 43

Total dose = 44s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69654 | 110-220 | 0.5 | 15/2 | 60K | 5NPLC | -300 | ON | H2O |
| 69655* 69656 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

* aborted

## Thick H2O dosing for 1080s (18 m) at 1e-7mbar

File: T69657
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 40K

Time start =  28

Time stop = 17m32s

Total dose = 1080+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69658 | 175-205 | 0.2 | 15/2 | 40K | 5NPLC | -100 | ON | H2O |
| 69659 | 110-220 | 1 |  |  |  | -300 |  | H2O |

## Anneal to 50 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69660* 69661 | 138-150 | 0.2 | 15/2 | 50K | 5NPLC | -450 | ON | H2O |

*aborted

## Anneal to 60 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69662 | 138-150 | 0.2 | 15/2 | 60K | 5NPLC | -450 | ON | H2O |
| 69663 | 200-220 | 0.5 |  |  |  | -300 |  |  |

## Anneal to 70 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69664 | 138-150 | 0.2 | 15/2 | 70K | 5NPLC | -450 | ON | H2O |

## Anneal to 80 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69665 | 138-150 | 0.2 | 15/2 | 80K | 5NPLC | -450 | ON | H2O |

## Anneal to 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69666 | 138-150 | 0.2 | 15/2 | 90K | 5NPLC | -450 | ON | H2O |

## Anneal to 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69667 | 138-150 | 0.2 | 15/2 | 100K | 5NPLC | -450 | ON | H2O |

## Anneal to 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69668 | 138-150 | 0.2 | 15/2 | 120K | 5NPLC | -450 | ON | H2O |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69669 | 138-150 | 0.2 | 15/2 | 120K | 5NPLC | -450 | ON | H2O |

Deosrbing?

## Repeat at 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69669 | 138-150 | 0.2 | 15/2 | 120K | 5NPLC | -450 | ON | H2O |

Yes desorbing at 140 K

# Saturday 28th May 2022 – Deposition at 70 K

08:17	6e-8 mbar	T = 271.3 K	cryo ON

09:30	only just realized there is no beam

10:30 	beam back

11:00	8e-9mbar	T = 70 K

## BKG scans at 70 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69671* 69672 | 110-220 | 0.5 | 15/2 | 70K | 5NPLC | -300 | ON | Clean MgF2 |
| 69673* 69674 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69675 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

*Aborted beam in decay 40 mA annnnd needed more coffee

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69676
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 70K

Time start =  29

Time stop = 44

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69677 | 110-220 | 0.5 | 15/2 | 70K | 5NPLC | -300 | ON | H2O |
| 69678 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69679
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 70K

Time start =  31s

Time stop = 46s

Total dose =15 + 15 +15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69679* 69680 | 110-220 | 0.5 | 15/2 | 70K | 5NPLC | -300 | ON | H2O |
| 69681 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

*aborted

## Dep 3: H2O dosing for 15s at 1e-7mbar

File: T69682
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 70K

Time start =  44s

Time stop = 59s

Total dose =15 + 15 + 15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69683 | 110-220 | 0.5 | 15/2 | 70K | 5NPLC | -300 | ON | H2O |
| 69684 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## H2O dep 4: dosing for 15s at 1e-7mbar

File: T69685
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 70K

Time start =  33s

Time stop = 48s

Total dose =15 + 15 + 15 + 15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69686 | 110-220 | 0.5 | 15/2 | 70K | 5NPLC | -300 | ON | H2O |
| 69687* 68688 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick dep: H2O dosing for 30min  at 1e-7mbar

File: T69689
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 70K

Time start =  27 s

Time stop = 30 m 27s

Total dose =15 + 15 + 15 + 15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69690 69691 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 80 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69692 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69693 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69694 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69695 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69696 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 160 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69697 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Repeat at 160 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69698 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

## Anneal to 180 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69699 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

Desorbed - 69700

# Saturday 28th May 2022 – Deposition at 120 K

## BKG scans at 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69701 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | Clean MgF2 |
| 69702 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69703 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

*Aborted beam in decay 40 mA annnnd needed more coffee

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69704
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 120K

Laser signal: 14uA

Time start =  30

Time stop = 45

Total dose = 15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69705 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | H2O |
| 69706 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |
| 69707 | 110-220 | 0.5 |  |  |  | -100 |  | H2O |

Ice appears to be changing?

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69708 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | Clean MgF2 |

Stopped growing… weird.

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69709
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 120K

Laser signal: 14uA

Time start =  30s

Time stop = 45s

Total dose = 15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69710 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | H2O |
| 69711 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1e-7mbar

File: T69712
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 120K

Laser signal: 13uA

Time start =  25 s

Time stop = 40 s

Total dose =15+15+ 15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69713 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | H2O |
| 69714 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1e-7mbar

File: T69715
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 120K

Laser signal: 13uA

Time start =  25 s

Time stop = 40 s

Total dose =15+15+ 15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69716* 69717 | 110-220 | 0.5 | 15/2 | 120K | 5NPLC | -300 | ON | H2O |
| 69718 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

*aborted

## Thick H2O dosing for 30 min at 1e-7mbar

File: T69719
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 120K

Laser signal: 13uA

Time start =  31 s

Time stop = 30m 31 s

Total dose =15+15+ 15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69720* 69721 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

*aborted

## Anneal to 120 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69722 | 175-205 | 0.2 | 15/2 | 70 K | 5NPLC | -100 | ON | H2O |

# Sunday 29th May – Deposition at 30 K

08:16 	P = 2e-8 mbar	T = 269.9 K	cryo ON

09:53	P = 5e-9		T = 30 K

## BKG scans at 30 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69723 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | Clean MgF2 |
| 69724* 69725* 69726 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69727 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

*aborted

## Dep 1: H2O dosing for 11s at 1-4e-7mbar (a bit of wild dep)

File: T69728
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 15 uA

Time start =  38 s

Time stop = 49

Total dose =11 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69729 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69730 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 1: H2O dosing for 15s at 1-4e-7mbar

File: T69731
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14.4 uA

Time start =  34 s

Time stop = 49 s

Total dose =11 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69732 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69733 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1-4e-7mbar

File: T69735
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 13.4 uA

Time start =  28 s

Time stop =  43 s

Total dose =11+15+15 s  dose

69734 aborted

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69736 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69737 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1-4e-7mbar

File: T69738
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12.3 uA

Time start =  28 s

Time stop =  43 s

Total dose =11+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69739 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69740 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 5: H2O dosing for 15s at 1-4e-7mbar

File: T69741
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12.3 uA

Time start =  28 s

Time stop =  43 s

Total dose =11+15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69742 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69743 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 30 min at 1-4e-7mbar

File: T69744
P before deposition: 4e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12.3 uA

Time start =  30 s

Time stop =  30 m 30 s

Total dose =11+15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69745 | 175-205 | 0.2 | 15/2 | 30K | 5NPLC | -100 | ON | H2O |

# Sunday 29th May – Deposition at 90 K

## BKG scans at 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69746 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | Clean MgF2 |
| 69747 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69748 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Dep 1: H2O dosing for 15s at 1.1e-7mbar

File: T69749
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14 uA

Time start =  34 s

Time stop =  49 s

Total dose =15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69750* 69751* 69752 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69753 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

*aborted

## Dep 2: H2O dosing for 15s at 1.1e-7mbar

File: T69754
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14.5 uA

Time start =   28 s

Time stop =  43 s

Total dose =15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69755 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69756 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1.1e-7mbar

File: T69757
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14.3 uA

Time start =   29 s

Time stop =  44 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69758 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69759 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1.1e-7mbar

File: T69760
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14.3 uA

Time start =   29 s

Time stop =  44 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69761 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69762 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 5: H2O dosing for 15s at 1.1e-7mbar

File: T69763
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12.5 uA

Time start =  30  s

Time stop =  43 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69764 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69765 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 30min at 1.1e-7mbar

File: T69766
P before deposition: 9e-9 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12 uA

Time start =  30  s

Time stop =  30min 30 s

Total dose =15+15+15+15+15s + 30min  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69767 | 175-205 | 0.2 | 15/2 | 30K | 5NPLC | -100 | ON | H2O |

20:57	cryo OFF

# Monday 30th May 2022 – Deposition at 80 K

08:00	p = 1.4e-8 mbar	T = 276.5 K

Turn off gauge and monitor diode signal while JSn is doing some tests in the ring. Problems on the AMO beamline with the new settings applied to the ring on Friday the 20th May which gives a higher lifetime.

REMEMBER TO RESET Keithley to correct settings for scanning. DONE!!

09:30	CRYO ON.

14:27	P=6e-8 mbar	T=80K

## BKG scans at 80 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69768 | 110-220 | 0.5 | 15/2 | 80K | 5NPLC | -300 | ON | Clean MgF2 |
| 69769 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69770 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Gas line dep fail

File: T69771.dat

69772 aborted
I just deposited ‘vacuum’ from the gas line… idiot. Maybe more caffeine is required.

What is it? Pressure increase to 1e-6 mbar around 117-120 K – has it desorbed?

69771 and 69772 ABORTED.

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69773 | 110-220 | 0.5 | 15/1 | 80K | 5NPLC | -300 | ON | H2O |
| 69774 |  |  | 15/2 | Heating |  |  |  | H2O |
| 69775 |  |  |  | Heating |  |  |  | H2O |

Something on the surface in 69773, fortunately only very low absorbance.

Started heating on the substrate and then measured both 69774 and 75 while the heater was on.

Apparently this does not work and gives a false signal on the diode, possibly giving off some light while on, such that there is current measured on the diode below the onset at 114 nm.

Stopped heating on the substrate and then measure again.

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69777 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69778 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |
| 69779 | 175-205 | 0.2 |  |  |  | -100 |  | H2O |

Turned combi gauge on after bkg scans and pressure at 1e-7 mbar but decreasing. Strange.

## Dep 1: H2O dosing for 15s at 1.1e-7mbar

File: T69780
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 12.5 uA

Time start =  26 s

Time stop =  41 s

Total dose =15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69781 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69782 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1.1e-7mbar

File: T69783
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 30K

Laser signal: 14.3 uA

Time start =  30 s

Time stop =  45 s

Total dose =15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69784 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69785 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1.1e-7mbar

File: T69786
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 80K

Laser signal: 14.3 uA

Time start =  24 s

Time stop =  39 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69787 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69788 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1.1e-7mbar

File: T69789
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 80K

Laser signal: 13.3 uA

Time start =  22 s

Time stop =  37 s

Total dose =15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69790 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69791 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 5: H2O dosing for 15s at 1.1e-7mbar

File: T69792
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 80K

Laser signal: 13.3 uA

Time start =  25 s

Time stop =  40 s

Total dose =15+15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69793 | 110-220 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |
| 69794 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 30 min at 1.1e-7mbar

File: T69795
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 80K

Laser signal: 11.3 uA

Time start =  29 s

Time stop =  30min24 s

Total dose =15+15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69796 | 175-205 | 0.2 | 15/2 | 80 K | 5NPLC | -100 | ON | H2O |

No ice?

## Even thicker H2O dep: dosing for 3 min at 1.1e-6mbar

File: T69795
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 80K

Laser signal: 11.3 uA

Time start =  35 s

Time stop =  3 min58 s

Total dose =15+15+15+15+15 + 30min +3 min(1e-6mbar) s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69800 | 110-175 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |

No peak for 190nm peak compared to other deps… puzzling.

# Tuesday 31st May 2022 – Deposition at 20 K (repeat)

07:40	p = 1.4e-8 mbar	T = 269.3 K

CRYO ON

09:23	p = 4 e-9 mbar	T = 19.2 K

## BKG scans at 19.1 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69801 | 110-220 | 0.5 | 15/2 | 20K | 5NPLC | -300 | ON | Clean MgF2 |
| 69802* 69803 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69804 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

*aborted

## Dep 1: H2O dosing for 15s at 1.1e-7mbar

File: T69805
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 14.4 uA

Time start =  33 s

Time stop =  48 s

Total dose =15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69806 | 110-220 | 0.5 | 15/2 | 19.1 K | 5NPLC | -300 | ON | H2O |
| 69807* 69808 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1.1e-7mbar

File: T69809
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 14. uA

Time start =  35 s

Time stop =  50 s

Total dose =15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69810* 69811 | 110-220 | 0.5 | 15/2 | 19.1 K | 5NPLC | -300 | ON | H2O |
| 69812 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

*aborted

## Dep 3: H2O dosing for 15s at 1.1e-7mbar

File: T69813
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 14. uA

Time start =  36 s

Time stop =  51 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69814 | 110-220 | 0.5 | 15/2 | 19.1 K | 5NPLC | -300 | ON | H2O |
| 69815 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1.1e-7mbar

File: T69816
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 14. uA

Time start =  26 s

Time stop =  41 s

Total dose =15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69817 | 110-220 | 0.5 | 15/2 | 19.1 K | 5NPLC | -300 | ON | H2O |
| 69818 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 5: H2O dosing for 16s at 1.1e-7mbar

File: T69821
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 10.6 uA

Time start =  30 s

Time stop =  46 s

Total dose =15+15+15+15s  dose

*69819, 69820 aborted

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69822 | 110-220 | 0.5 | 15/2 | 19.1 K | 5NPLC | -300 | ON | H2O |
| 69823 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 16s at 1.1e-7mbar

File: T69824
P before deposition: 3e-8 mbar

Temperature of substrate during deposition: 19.1K

Laser signal: 9.6 uA

Time start =  29 s

Time stop =  30m29s

Total dose =15+15+15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69825* 69826 | 110-175 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |

*aborted

## Warming to 250 K

Left the combi gauge on for a bit.

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69827 | 110-175 | 0.5 | 15/2 | 80 K | 5NPLC | -300 | ON | H2O |

# Tuesday 31st May 2022 – Deposition at 50 K (repeat)

## BKG scans at 50 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69828 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | Clean MgF2 |
| 69829* | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69830 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

*something weird in d02 file.. do not use

## Dep 1: H2O dosing for 15s at 1.1e-7mbar

File: T69831
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 10.6 uA

Time start = 30  s

Time stop =  45 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69832 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69833 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1.8e-7mbar

File: T69834
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 10.6 uA

Time start = 31  s

Time stop =  46 s

Total dose =15+15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69835 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69836 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1.8e-7mbar

File: T69837
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 12.6 uA

Time start = 28  s

Time stop =  43 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69838 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69839 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 15s at 1.8e-7mbar

File: T69840
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 11.6 uA

Time start = 31  s

Time stop =  46 s

Total dose =15+15+15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69841 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69842 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 5: H2O dosing for 15s at 1.8e-7mbar

File: T69843
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 9. uA

Time start = 27  s

Time stop =  43 s

Total dose =15+15+15+15+16s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69844 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |
| 69845 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 30min at 1.8e-7mbar

File: T69846
P before deposition: 8e-9 mbar

Temperature of substrate during deposition: 50K

Laser signal: 9. uA

Time start = 27  s

Time stop =  43 s

Total dose =15+15+15+15+16s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69847 | 175-205 | 0.2 | 15/2 | 50K | 5NPLC | -100 | ON | H2O |

## Anneal to 70 K (pore collapse?)

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69848 | 175-205 | 0.2 | 15/2 | 50K | 5NPLC | -100 | ON | H2O |

## Anneal to 130 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69849 | 175-205 | 0.2 | 15/2 | 50K | 5NPLC | -100 | ON | H2O |
| 69850 |  |  |  |  |  |  |  |  |

## Anneal to 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69851 | 175-205 | 0.2 | 15/2 | 50K | 5NPLC | -100 | ON | H2O |

21:38	cryo OFF

# Wednesday 1st June 2022 – Deposition at 110 K

08:00	p = 1.3e-8 mbar	T = 269.5 K

CRYO ON

09:20	P = 1e-8 mbar	T = 110 K

## BKG scans at 110 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69852 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | Clean MgF2 |
| 69853 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69854 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Dep 1: Pump oil again… Why???

File: T69855

69856

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69857 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | oil |

Destroying – scans taken while heating

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69858 | 110-220 | 0.5 | 15/2 | 190- | 5NPLC | -300 | ON | oil |

# Wednesday 1st June 2022 – Deposition at 135 K

11:51	restart

P = 	T = 135 K

## BKG scans at 135 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69860 | 110-220 | 0.5 | 15/2 | 135K | 5NPLC | -300 | ON | Clean MgF2 |
| 69861* ABORTED | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69862 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69863 | 175-205 | 0.2 |  |  |  | -100 |  |  |

## Dep 1: H2O dosing for 15s at 0.8-4e-7mbar (very dodgy)

File: T69864
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 24  s

Time stop =  39 s

Total dose =15s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69865 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

Nothing.

## Dep 2: H2O dosing for 30s at 1-2e-7mbar

File: T69866
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 25  s

Time stop =  55 s

Total dose =15+30s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69867 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Dep 3: H2O dosing for 120s at 1-2e-7mbar

File: T69868
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 31  s

Time stop =  2m31 s

Total dose =15+30+150s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69869 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Dep 4: H2O dosing for 10 mins at 1-2e-7mbar

File: T69870
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 46  s

Time stop =  10m46s

Total dose =15+30+150s +10 min  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69871 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Dep 5: H2O dosing for 5 mins at 1-2e-7mbar

File: T69872
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 27  s

Time stop =  5m27s

Total dose =15+30+150s +10 min + 5min dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69873 69874 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Thick H2O dep: dosing for  mins at 1-2e-7mbar

29 s to 14:36 1e-7 mbar

14:36 onwards 1e-6

File: T69875
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 135K

Laser signal: 15 uA

Time start = 27  s

Time stop =  5m27s

Total dose =15+30+150s +10 min + 5min dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69877 | 175-205 | 0.2 | 15/2 | 135K | 5NPLC | -100 | ON | H2O |

69876, 69878* aborted?

69878

# Wednesday 1st June 2022 – Deposition at 140 K

11:51	restart

P = 	T = 135 K

## BKG scans at 140 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69880 | 110-220 | 0.5 | 15/2 | 135K | 5NPLC | -300 | ON | Clean MgF2 |
| 69881 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep 1: H2O dosing for 120s at 1e-7mbar

File: T69883

69882* aborted
P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 49  s

Time stop =  2m49 s

Total dose =120s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69884 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

Nothing there

## Dep 2: H2O dosing for 240s at 1e-7mbar

File: T69885

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 35  s

Time stop =  3m35 s

Total dose =120 + 240s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69886 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

Not sicking

## Dep 3: H2O dosing for 60s at 5e-7mbar

File: T69887

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 26  s

Time stop =  1m26 s

Total dose =120 + 240s +60s dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69888 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Dep 4: H2O dosing for 180s at 5e-7mbar

File: T69889

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 35  s

Time stop =  1m05 s

Total dose =120 + 240s +60+180 +30dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69890 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Dep 5: H2O dosing for 30s at 5e-7mbar

File: T69891

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 35  s

Time stop =  1m05 s

Total dose =120 + 240s +60+180 +30dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69892 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

## Thick H2O dep: dosing for 20min at 5e-7mbar

File: T69893

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 140K

Laser signal: 15 uA

Time start = 35  s

Time stop =  1m05 s

Total dose =120 + 240s +60+180 +30+20

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69894 | 110-220 | 0.5 | 15/2 | 30K | 5NPLC | -300 | ON | H2O |

22:56 	cryo OFF

# Thursday 2nd June 2022 – Deposition at 110 K

07:45	p = 1.4e-8 mbar	T = 270.6 K

CRYO ON

09:32	p = 8e-9 mbar	T = 110 K

## BKG scans at 110 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69895 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |
| 69896 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 69897 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69898

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 53  s

Time stop =  15 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69899 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69900 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69901

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 45  s

Time stop =  60 s

Total dose =15 +15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69902 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69903 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1e-7mbar

5.15e-7 mbar from ~100 s

File: T69904

P before deposition: 6e-9 mbar

Temperature of substrate during deposition: 110K

Laser signal: 13.5 uA

Time start =   31s

Time stop =   51s

Total dose =15 +15 +15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69905 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69906 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick dep H2O: dosing for 9min  at 5e-7mbar

5.15e-7 mbar from ~100 s

File: T69907

P before deposition: 6e-9 mbar

Temperature of substrate during deposition: 110K

Laser signal: 13.5 uA

Time start =   30s

Time stop =   9m30s

Total dose =15 +15 +15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69908 | 175-205 | 0.2 | 15/2 | 110K | 5NPLC | -100 | ON | H2O |

## Even thicker dep H2O: dosing for 9min  at 5e-7mbar

1e-6-7 mbar from ~190 s

File: T69909

P before deposition: 6e-9 mbar

Temperature of substrate during deposition: 110K

Laser signal: 13.5 uA

Time start =   34s

Time stop =   6m34s

Total dose =15 +15 +15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69910 | 175-205 | 0.2 | 15/2 | 110K | 5NPLC | -100 | ON | H2O |

Clean?

69911.d01 - yep

# Thursday 2nd June 2022 – Deposition at 100 K

## BKG scans at 100 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69912 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |
| 66913 | ABORTED |  |  |  |  |  |  |  |
| 66914 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |
| 66915 | 175-205 | 0.2 |  |  |  | -100 |  | Clean MgF2 |

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69916

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 53  s

Time stop =  15 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66917 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69918 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69919

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 28  s

Time stop =  44 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66920 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69921 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1e-7mbar

File: T69922

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 13.7 uA

Time start = 36  s

Time stop =  25 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66923 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |
| 69924 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Thick H2O dep: dosing for 6min40s at 5e-7mbar

File: T69925

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 13.7 uA

Time start = 29  s

Time stop =  7min09 s

Total dose = s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69926 | 175-205 | 0.2 | 15/2 | 100K | 5NPLC | -100 | ON | H2O |

# Thursday 2nd June 2022 – Deposition at 90 K

## BKG scans at 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69928 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |
| 66929 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep 1: H2O dosing for 10s at 5e-8mbar

File: T69930

Deposited nothing – air?

Idiot idiot idiot

69931* aborted

69932

## BKG scans at 90 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69934 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |
| 69935 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep1: H2O dosing for 10s at 5e-8mbar

File: T69936

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 13.7 uA

Time start = 39  s

Time stop =  49 s

Total dose = s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69937 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep2: H2O dosing for 10s at 5e-8mbar

File: T69938

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 13.7 uA

Time start = 33  s

Time stop =  43 s

Total dose = 10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69939 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

Only scanning once

## Dep3: H2O dosing for 10s at 5e-8mbar

File: T69940

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 13.7 uA

Time start = 12  s

Time stop =  49 s

Total dose = 10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69941 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | CH2O |

## Dep 4: H2O dosing for 10s at 5e-8mbar

File: T69942

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 14.3 uA

Time start = 24  s

Time stop =  34 s

Total dose = 10+10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69943 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 5: H2O dosing for 10s at 5e-8mbar

File: T69944

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 14.3 uA

Time start =  32 s

Time stop =  42 s

Total dose = 10+10+10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69945 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 6: H2O dosing for 10s at 5e-8mbar

File: T69944

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 14.3 uA

Time start = 34  s

Time stop =  44 s

Total dose = 10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69947 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 7: H2O dosing for 10s at 5e-8mbar

File: T69948

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 13.3 uA

Time start = 30  s

Time stop =  40 s

Total dose = 10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69949 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 7: H2O dosing for 10s at 5e-8mbar

File: T69948

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 13.3 uA

Time start = 29  s

Time stop =  39 s

Total dose = 10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69950 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 8: H2O dosing for 10s at 5e-8mbar

File: T69951

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 12.8 uA

Time start = 26  s

Time stop =  36 s

Total dose = 10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69952 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 9: H2O dosing for 10s at 5e-8mbar

File: T69953

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Laser signal: 12.8 uA

Time start = 26  s

Time stop =  36 s

Total dose = 10+10+10s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69952 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 10: H2O dosing for 10s at 5e-8mbar

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Dose: 10s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69953 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

## Dep 11: H2O dosing for 10s at 5e-8mbar

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 90K

Dose: 10s

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69954* 69955 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | H2O |

*aborted

01:44	cryo Off

# Friday 3rd June 2022 – Deposition at 130 K

07:40	p = 2.5e-8 mbar	T = 247.2 K

CRYO ON

09:12	T= 18.1 K P = 1.7e-8 mbar

09:36	P = 6e-9 mbar	T = 80 K

## BKG scans at 80 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69957 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |

Change of plan

## BKG scans at 130 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69958 | 110-220 | 0.5 | 15/2 | 110K | 5NPLC | -300 | ON | Clean MgF2 |
| 66959* 66960 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

*aborted

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69961

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 1min39  s

Time stop =  1min54 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69962 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
|  | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 29s at 1e-7mbar

File: T69963

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 28  s

Time stop =  57 s

Total dose =15+29 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69964 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69965 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 29s at 1.1e-7mbar

File: T69966

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 30  s

Time stop =  58 s

Total dose =15+29 +29 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69967 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69968 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 4: H2O dosing for 29s at 1.1e-7mbar

File: T69969

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 30  s

Time stop =  58 s

Total dose =15+29 +29 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69970 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69971 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Annealing to 132 K (1 scan)

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69972 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Annealing to 134 K (1 scan)

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69973 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Annealing to 138 K (1 scan)

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69974 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Annealing to 141 K (1 scan)

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69975 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

# Friday 3rd June 2022 – Deposition at 125 K

## BKG scans at 125 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69976 | 110-220 | 0.5 | 15/2 | 125K | 5NPLC | -300 | ON | Clean MgF2 |
| 69977 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

*aborted

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69978

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start =  1min 25 s

Time stop =   1min40s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69979 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 699 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

*Dohhh, left the gauge on in the BKG and didn’t check***

## Laser dep experiment: H2O dosing for 30 min at 1e-7mbar

File: T6997

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start =  1min 25 s

Time stop =   1min40s

Total dose =15 s  dose

17:35	cryo OFF

# Saturday 4th June – Deposition at 125 K

07:44	p = 2e-8 mbar	 T = 278.9 K 	cryo ON

## BKG scans at 125 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69982* 69983 | 110-220 | 0.5 | 15/2 | 125K | 5NPLC | -300 | ON | Clean MgF2 |
| 69984 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

*aborted

## Dep 1: H2O dosing for 15s at 1e-7mbar

File: T69985

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 42 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69986 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69987 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 2: H2O dosing for 15s at 1e-7mbar

File: T69988

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 30 s

Time stop =  45 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69989 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69990 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Dep 3: H2O dosing for 15s at 1e-7mbar

File: T69991

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 42 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69992 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

Intensity increasing – rescan a few times

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69993 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 69994 |  |  |  |  |  |  |  |  |
| 69995 |  |  |  |  |  |  |  |  |
| 69996 |  |  |  |  |  |  |  |  |

## Thick dep H2O: dosing for 15s at 1e-6mbar

File: T69997

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 42 s

Time stop =  57 s

Total dose =15 s  dose

Clean substrate – 69998

# Saturday 4th June – Deposition at 127.5 K

## BKG scans at 127.5 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 69999 | 110-220 | 0.5 | 15/2 | 125K | 5NPLC | -300 | ON | Clean MgF2 |
| 70000 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep 1: H2O dosing for 15s at 1e-7 ?? mbar

File: T70002

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 37 uA

Time start = 42 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 670003 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

Need more.

## Dep 2: H2O dosing for 15s at 1e-7 ?? mbar

File: T70004

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 127.5K

Laser signal: 14 uA

Time start = 49 s

Time stop =  1min04 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 670005 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Thick H2O dosing for 15s at 1e-7 mbar

File: T70006

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 127.5K

Laser signal: 14 uA

Time start = 49 s

Time stop =  1min04 s

Total dose =15 s  dose

# Saturday 4th June – Deposition at 137.5 K

## BKG scans at 127.5 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70008 | 110-220 | 0.5 | 15/2 | 125K | 5NPLC | -300 | ON | Clean MgF2 |
| 70009 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep 1: H2O dosing for 30s at 1e-7 mbar

File: T70010

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 35 s

Time stop =  55 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70011* 70012 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Dep 2: H2O dosing for 30s at 5e-7 mbar

File: T70013

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 27 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70014 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Dep 3: H2O dosing for 30s at 5e-7 mbar

File: T70015

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 27 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70016 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Dep 4: H2O dosing for 30s at 5e-7 mbar

File: T70017

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 27 s

Time stop =  57 s

Total dose =15 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70018 70019 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 70020 | 138-150 |  |  |  |  |  |  |  |

## Thick H2O dep: dosing for 5min at 5e-7 mbar

File: T70020

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 32s

Time stop =  5min32 s

Total dose =15 s  dose

# Sunday 5th June 2022 – Deposition at 80 K

07:45	P = 2e-8 mbar	T = 275 K

## BKG scans at 127.5 K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70022 | 110-220 | 0.5 | 15/2 | 125K | 5NPLC | -300 | ON | Clean MgF2 |
| 70023 | 138-150 | 0.2 |  |  |  | -450 |  | Clean MgF2 |

## Dep 1: H2O dosing for 45s at 1e-7 mbar

File: T70024

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 33 s

Time stop = 1min18 s

Total dose =45 s  dose

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70025 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |
| 70026 | 138-150 | 0.2 |  |  |  | -450 |  | H2O |

## Anneal to 90K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70027 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 100K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70028 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 110K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70029 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 120K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70030 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 125K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70031 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 130K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70032 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Anneal to 140K

| File |  range |  step | #av/#sc | Temp/K | Keithley | Table/kstp | PMTVac | Sample |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 70033 | 110-220 | 0.5 | 15/2 | 130K | 5NPLC | -300 | ON | H2O |

## Substrate clean – 70034

## Thick dep H2O: dosing for  at 1e-7 mbar

File: T70035

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 24 s

Time stop = 7min

Total dose =45 s  dose

## 50 K Thick dep H2O: dosing for 460s  at 1e-7 mbar

File: T7003

P before deposition: 2e-8 mbar

Temperature of substrate during deposition: 110K

Laser signal: 14 uA

Time start = 24 s

Time stop = 7min

Total dose =45 s  dose